Console V1.00

CONSOLE.EXE is a remote console server for NNA BBS software V4.1.4 and later.

It allows the BBS to be operated at a remote location within the same

building using a RS232 null modem cable.

CONSOLE.EXE mimics the BBS in every way including the triggering of the

screen blanker. A spare RS232 port is used at both end that are NOT used by

any other software. CONSOLE.EXE runs in DOS or in a DesQview window and

should run in a DOS window in Windows 95/98 without any problem. Several

copies of CONSOLE.EXE can be run at the same time in DesQview or Windows

providing each copy is using a separate COM port, IRQ and Base Address.

It is important that the RS232 ports used are not used by anything else i.e.

BPQ, mouse or any other application. To connect CONSOLE  to the BBS you will

need a well screened null modem cable. The connections of which are as

follows:-

25 way to 25 way

Pin end one		       Pin end two

2      <  -------------------- >        3     TxD - RxD

3      < --------------------- >        2     RxD - TxD

4      < --------------------- >        5     RTS - CTS

5      <  -------------------- >        4     CTS - RTS

6      <  -------------------- >        20    DSR - DTR

7      <  -------------------- >        7     Ground

20     < --------------------- >        6     DTR - DSR

25 Way to 9 way

Pin 25 way                              Pin 9 Way

2      < --------------------- >    2   TxD - RxD

3      < --------------------- >    3   RxD - TxD

4      < --------------------- >    8   RTS - CTS

5      < --------------------- >    7   CTS - RTS

6      < --------------------- >    4   DSR - DTR

7      < --------------------- >    5  Ground

20     < --------------------- >    6   DTR - DSR

9 Way to 9 Way

Pin end one                           Pin end two

2   < ----------------------- >    3   RxD - TxD

3   < ------------------------>    2   TxD - RxD

4   < ----------------------- >    6   DTR - DSR

5   < ----------------------- >    5   Ground

6   < ----------------------- >    4   DSR - DTR

7   < ----------------------- >    8   RTS - CTS

8   < ----------------------- >    7   CTS - RTS

The cable used must be well screen and length kept to a minimum. As I recall

the maximum length for a RS 232 cable is about 25 ft.

CONFIG.CON

CONSOLE.EXE needs the following configuration (CONFIG.CON) file which MUST

reside in the same directory as CONSOLE.EXE as follows:-

; General setup

GB7NNA                          ; BBS call sign

1                               ; COM port

4                               ; COM port IRQ

3F8                             ; COM port base address

38400                           ; COM port speed

; Display bits

1                               ; Main window background colour

14                              ; Text colour for top line of main Window.

2                               ; Monitor window background colour

15                              ; Text colour for monitor window

7                               ; Status lines background colour

0                               ; Status lines text colour

0                               ; Background colour for selected port

14                              ; Text colour for selected port

5                               ; Bottom `menu' background colour

7                               ; Bottom `menu' text colour

NO                              ; Write directly to screen

CONFIG.CON details

--------------------------

BBS call sign is the call sign of the BBS you are linking to. The COM port,

IRQ and Base address are the addresses for the COM port you are using. The

speed it the BAUD rate and this MUST be the same at both ends i.e. the BBS

end and the CONSOLE end. Do not attempt to use very high speeds over long

distances. If you find you are getting data loss try reducing the baud rate.

The Display bits are the same as the BBS display bits but of course can be

different from the BBS set up if you so wish.

All the commands are of course the same as the BBS with the addition ALT X

to close the console down and CTRL I to insert a file into the text editor

from the Console instead of the BBS.

Remember that when running CONSOLE.EXE you are talking to the BBS and not the

machine running CONSOLE.EXE. This takes a bit of getting used to.

If for any reason the link to the BBS is lost, CONSOLE will make a number of

attempts to re connect. It does not matter which is booted first but it is

advisable to configure and boot the BBS first.